Exam 1 Review

1. Definition and Components of a Biosensor

Key Concepts

- Biosensor Definition: A biosensor is an analytical device that integrates a biological or biomimetic recognition element with a transducer to detect a target molecule (analyte) and convert this interaction into a measurable signal.

- Biosensors can be used for applications in medical diagnostics, food safety, environmental monitoring, and biodefense.

Six Key Components of a Biosensor:

- Target Molecule (Analyte) – The specific substance that the biosensor is designed to detect (e.g., glucose, pathogens, toxins, biomarkers).

- Recognition Element – A biological or biomimetic component that selectively binds to the target molecule (e.g., enzymes, antibodies, nucleic acids, aptamers, imprinted polymers).

- Transducer – Converts the biochemical interaction between the recognition element and the target molecule into a measurable signal (e.g., electrochemical, optical, piezoelectric, thermal).

- Detector/Amplifier – Processes and amplifies the signal from the transducer, increasing sensitivity and accuracy.

- Display/Output – Converts the processed signal into a readable format (e.g., digital readout, colorimetric change, graphical representation).

- End-User/Application – The specific use of the biosensor, defining how and where it is applied (e.g., medical diagnostics, food safety, environmental monitoring, biodefense).  This also helps determine the required level of ease of use in the biosensor design, depending on whether the end user is a trained technician or a layperson using the device at home.

Biosensors & Bioelectronics Journal Definition

According to Biosensors & Bioelectronics, a biosensor must incorporate at least one of the following:

- A Biological Material – e.g., tissue, microorganisms, organelles, cell receptors, enzymes, antibodies, nucleic acids, natural products.

- A Biologically Derived Material – e.g., recombinant antibodies, engineered proteins, aptamers.

- A Biomimetic Material – e.g., synthetic receptors, biomimetic catalysts, combinatorial ligands, imprinted polymers.

Review Questions

- What are the six components of a biosensor, and what role does each play?

- How do enzyme-based, antibody-based, and nucleic acid-based biosensors differ?

- What three materials must a sensor include to be considered a biosensor?

- How does the transducer type influence biosensor sensitivity and selectivity?

- What are the advantages and limitations of biosensors compared to traditional chemical sensors?

- What are some real-world applications of biosensors in medicine and environmental monitoring?

- Given a glucose biosensor, identify the analyte, recognition element, transducer, signal processing component, and output. Explain how changing the transducer (e.g., optical vs electrochemical) would affect sensor performance.

2. Enzyme-Linked Immunosorbent Assay (ELISA)

- ELISA (Enzyme-Linked Immunosorbent Assay) is a biochemical technique used to detect the presence of an antigen or antibody in a sample.

- Antigen-Antibody Interactions: ELISA relies on highly specific antigen-antibody binding to detect biomolecules.

- Types of ELISA:

  - Direct ELISA
In a direct ELISA, the antigen is immobilized on the plate and directly detected using an enzyme-linked primary antibody. This format is fast and simple with fewer steps and reduced risk of cross-reactivity, but it has lower sensitivity because there is no signal amplification and each primary antibody must be enzyme-labeled. The detection signal is directly proportional to the analyte concentration.

  - Indirect ELISA
In an indirect ELISA, the antigen is immobilized on the plate, a primary antibody binds to the antigen, and an enzyme-linked secondary antibody binds to the primary antibody to amplify the signal. This approach provides higher sensitivity and greater flexibility because a single labeled secondary antibody can be used with many primary antibodies, although it requires additional steps and may introduce higher background signal. The detection signal is directly proportional to the analyte concentration.

  - Sandwich ELISA
In a sandwich ELISA, the target antigen is captured between a surface-immobilized capture antibody and an enzyme-linked detection antibody. This format offers high specificity and sensitivity and performs well in complex samples, but it requires two well-matched antibodies and is more complex and costly to implement. The detection signal is directly proportional to the analyte concentration.

  - Competitive ELISA
In a competitive ELISA, the target analyte in the sample competes with a labeled antigen for a limited number of antibody binding sites. This format is useful for small molecules and cases where only one antibody is available, but the signal is less intuitive and typically less sensitive than sandwich ELISA. The detection signal is inversely proportional to the analyte concentration.

- Detection Methods:

  - Colorimetric – Uses enzyme-linked reactions for color change.

  - Fluorescent – Uses fluorescent-labeled antibodies for high sensitivity.

  - Electrochemical ELISA – Measures current or voltage changes upon enzyme reaction.

- Key Enzymes Used in ELISA:

  - Horseradish peroxidase (HRP)

  - Alkaline phosphatase (ALP)

  - β-galactosidase

Horseradish peroxidase (HRP) catalyzes oxidation reactions in the presence of hydrogen peroxide and substrates such as TMB, producing a strong color change, fluorescence, or light emission, which makes HRP the most widely used ELISA enzyme due to its high sensitivity and fast kinetics. Alkaline phosphatase (ALP) removes phosphate groups from substrates such as pNPP, generating colored, fluorescent, or luminescent products and is valued for its stability and lower background in some assays. β-galactosidase cleaves galactoside substrates to generate detectable products, including colorimetric and fluorescent signals, but is used less frequently due to its larger size and slower reaction kinetics.

- Applications of ELISA:

  - Disease diagnostics (HIV, COVID-19).

  - Food safety testing (detecting allergens, pathogens).

  - Drug screening and toxicology studies.

  - Environmental monitoring (detecting contaminants in water, soil).

Review Questions

- What is ELISA, and how does it work?

- What are the four types of ELISA, and how do they differ?

- What is the role of antibodies in ELISA?

- How does competitive ELISA differ from sandwich ELISA?

- What is the purpose of the enzyme labels (HRP, ALP, β-galactosidase) in ELISA?

- Why are wash steps critical in ELISA procedures?

- How does fluorescent ELISA compare to colorimetric ELISA in sensitivity?

- What factors affect ELISA specificity and sensitivity?

- What are the advantages of using electrochemical ELISA over traditional ELISA?

- Describe how a competitive ELISA works and draw a simple diagram showing antigen-antibody interactions.

- For each ELISA type (direct, indirect, sandwich, competitive), state whether signal increases or decreases with increasing analyte concentration.

# 3. Polymerase Chain Reaction (PCR)

## Key Concepts

### Purpose and Basic Mechanism

- Polymerase Chain Reaction (PCR) is a technique used to amplify specific DNA sequences, generating millions of copies from a small starting amount.

- The process is temperature-controlled and relies on Taq polymerase, a heat-stable enzyme.

### Three Major Steps of PCR

- Denaturation (~94–98°C) – The double-stranded DNA (dsDNA) is heated to high temperatures, causing the hydrogen bonds between the base pairs to break and separating the two strands.

- Annealing (~50–65°C) – Short DNA primers (complementary to the target DNA sequence) bind to the single-stranded template DNA, marking the starting point for new DNA synthesis.

- Extension (~72°C) – Taq polymerase synthesizes new DNA by adding nucleotides (dNTPs) to the primers, extending the DNA strand in the 5’ to 3’ direction.

## Each PCR thermal cycle ideally doubles the amount of target DNA, resulting in exponential amplification over successive cycles. In practice, PCR amplification is limited to a finite number of cycles—typically 30 to 40—because reaction efficiency decreases over time. As cycling continues, enzyme activity gradually declines due to thermal stress, dNTPs and primers become depleted, and accumulated reaction byproducts can inhibit polymerase activity. Together, these factors cause the amplification to plateau, preventing continued exponential DNA production.

## Other Types of PCR

### qPCR (Quantitative PCR) / Real-Time PCR

- Monitors DNA amplification in real-time using fluorescent dyes or fluorescent-labeled probes r

- Unlike traditional PCR, which only detects DNA at the endpoint, qPCR quantifies DNA concentration after each cycle. This allows qPCR to provide quantitative information about the initial amount of target DNA in a sample and hence generally has a higher sensitivity and dynamic range than traditional PCR.

- Eliminates the need for post-PCR gel electrophoresis, reducing contamination risk

- Fluorescence Detection Methods:

  - SYBR Green – Nonspecifically binds to double-stranded DNA and emits fluorescence when bound and is therefore simple and cost-effective.  However, because SYBR Green binds to all double-stranded DNA, including nonspecific products and primer-dimers, careful primer design and melt-curve analysis are required to ensure specificity.

  - TaqMan Probes – Specific fluorescent-labeled probes hybridize to the target DNA, increasing specificity.    TaqMan probes provide higher specificity by using a sequence-specific fluorescent probe that hybridizes to the target DNA between the forward and reverse primers. During the extension step, the 5′→3′ exonuclease activity of Taq polymerase cleaves the probe, separating the fluorophore from a quencher and producing a fluorescence signal that is directly proportional to the amount of target DNA amplified. Although TaqMan assays are more expensive than SYBR Green-based assays, they offer improved specificity, reduced background signal, and are well suited for multiplexed detection.

- Advantages of qPCR over traditional PCR:

  - Detects and quantifies DNA concentration dynamically.

  - Provides high sensitivity and real-time monitoring.

  - Reduces risk of contamination since post-PCR handling is minimized.

### RT-PCR (Reverse Transcription PCR)

- Used for RNA analysis, especially for detecting RNA viruses (e.g., SARS-CoV-2) or gene expression studies.

- Steps in RT-PCR:

  - Reverse Transcription (RT): RNA is converted into complementary DNA (cDNA) using the enzyme reverse transcriptase.

  - PCR Amplification: The cDNA is then amplified using traditional PCR.

- Key Differences Between RT-PCR and Regular PCR:

  - RT-PCR starts with RNA, while traditional PCR starts with DNA.

  - Requires an additional enzyme (reverse transcriptase) for RNA-to-DNA conversion.

Clarification:  qPCR refers to real-time quantitative PCR (DNA). RT-PCR refers to reverse transcription PCR (RNA → cDNA). Real-time RT-PCR combines both.

## Discovery of Taq Polymerase

- Taq polymerase was first discovered in 1969 in the thermophilic bacterium Thermus aquaticus, which thrives in the hot springs of Yellowstone National Park.

- Unlike regular DNA polymerase, Taq polymerase is heat-stable, allowing it to survive the high denaturation temperatures (~94–98°C) in PCR without degrading.

- This discovery revolutionized PCR technology, enabling the automation of DNA amplification without the need to add new polymerase after each cycle.

## Applications of PCR in Biosensors and Diagnostics

- Pathogen Detection: Used for detecting viruses, bacteria, and genetic mutations (e.g., COVID-19, HIV, tuberculosis).

- Forensic Analysis: Amplifies trace amounts of DNA from crime scenes for genetic fingerprinting.

- Genetic Testing: Used for diagnosing hereditary diseases and identifying genetic variants.

- Biosensing: PCR-based biosensors integrate DNA amplification with electrochemical or fluorescence-based detection, enabling rapid, sensitive diagnostics.

## Review Questions

- What are the three major steps of PCR, and what happens at each stage?

- Why is Taq polymerase used instead of regular DNA polymerase? Where was it first discovered in nature?

- What is the function of primers in PCR, and how do they influence amplification specificity?

- How do temperature variations control each stage of PCR?

- What are the key advantages of qPCR over traditional PCR?

- How does RT-PCR differ from conventional PCR, and why is it useful for detecting RNA viruses?

- What are amplicons, and why are they important in PCR?

- How does SYBR Green differ from TaqMan probes in qPCR detection?

- Why does Taq polymerase function at high temperatures, and how does this benefit PCR?

- What enzyme is used in RT-PCR to convert RNA into cDNA, and why is this step necessary?

- How is qPCR fluorescence intensity correlated to DNA amplification?

- What are the main applications of PCR in biosensor technology?

- What precautions must be taken to prevent contamination in PCR experiments?

- How does PCR help diagnose infectious diseases like COVID-19?

- What role do dNTPs (nucleotides) play in PCR?

- How does cycle threshold (Ct value) in qPCR help quantify DNA concentration?

4. Gel Electrophoresis (Focused on DNA & RNA Analysis)

## Key Concepts

- Gel Electrophoresis is a technique used to separate DNA and RNA molecules based on size and charge using an applied electric field.

- Principle: Negatively charged nucleic acids migrate through a gel matrix toward the positive electrode (anode).  Note DNA is negatively charged due to its phosphate backbone, which causes it to migrate toward the positively charged anode.

- Agarose Gel Electrophoresis:

  - Used primarily for DNA and RNA separation.

  - Lower resolution compared to polyacrylamide but easier to handle.

  - Typically stained with ethidium bromide (EtBr) or SYBR Green for visualization.

- Factors Affecting DNA/RNA Migration:

  - Agarose Concentration: Higher concentration = better separation of small fragments.

  - Voltage: Higher voltage increases speed but can distort bands.

  - Buffer System: Maintains pH and ionic strength for consistent migration.

- DNA Size Determination:

  - DNA Ladders (molecular weight markers) contain fragments of known sizes for comparison.

  - Smaller fragments migrate faster than larger ones.

- Common Applications:

  - Confirming PCR products.

  - Genotyping and forensic analysis.

  - Checking RNA integrity before downstream applications (e.g., RT-PCR).

- Protein Gell Electrophoresis (Note you can run gels for measuring proteins as well and even nanoparticles…we do not spend time discussing this in class so you will not be responsible for this in the Exam).

## Review Questions

- What is gel electrophoresis, and how does it separate DNA and RNA?

- Why does DNA migrate toward the positive electrode during electrophoresis?

- How does agarose concentration affect the resolution of DNA fragments?

- What is the purpose of using ethidium bromide or SYBR Green in DNA analysis?

- How does a DNA ladder help estimate fragment sizes?

- What happens if you apply too high a voltage to an agarose gel?

- Why is a buffer system (e.g., TAE or TBE) necessary for gel electrophoresis?

- How does gel electrophoresis confirm the success of a PCR reaction?

- What are the limitations of agarose gel electrophoresis compared to other DNA analysis techniques?

- Describe the step-by-step process of running an agarose gel for DNA analysis.

## 5. Mass Spectrometry (MS) – Basics

## Key Concepts

- Mass Spectrometry (MS) is an analytical technique used to determine the mass-to-charge ratio (m/z) of molecules.

- Principle:

  - Ionization – Converts molecules into charged particles.

  - Mass Analysis – Separates ions based on mass-to-charge ratio (m/z).

  - Detection – Measures ion abundance and generates a mass spectrum.

- Common Ionization Techniques:

  - Electron Impact (EI) – Used for volatile molecules in gas chromatography-mass spectrometry (GC-MS).

  - Electrospray Ionization (ESI) – Suitable for large biomolecules like nucleic acids.

  - Matrix-Assisted Laser Desorption/Ionization (MALDI) – Used in proteomics to analyze macromolecules.

- Interpreting Mass Spectra:

  - Molecular Ion Peak (M⁺) – Represents the entire molecule’s mass.

  - Fragmentation Patterns – Provide structural information about a molecule.

  - Isotopic Peaks – Help in identifying elements based on their natural isotopic distribution.

- Applications:

  - Metabolomics – Analyzing metabolic compounds in biological systems.

  - Drug Discovery – Identifying and quantifying pharmaceutical compounds.

  - Environmental Monitoring – Detecting contaminants in air, water, and soil.

## Review Questions

- What is mass spectrometry, and how does it work?

- How does ionization convert molecules into charged species?

- What is the difference between EI, ESI, and MALDI ionization techniques?

- What does the molecular ion peak (M⁺) represent in a mass spectrum?

- Why is mass spectrometry coupled with chromatography techniques (e.g., GC-MS, LC-MS)?

- What are the key advantages of using MS over traditional spectroscopic techniques?

- How does mass spectrometry help in drug discovery and environmental science?

- Briefly, describe the process of interpreting a simple mass spectrum and identifying key peaks.

## 6. Chromatography – History, Principles, and Applications

### Key Concepts

- Origins of Chromatography:

  - Developed in 1900 by Russian botanist Mikhail Tsvet, chromatography was initially used to separate plant pigments (chlorophyll, xanthophyll, and carotenoids) using column chromatography.

  - The name “chromatography” comes from the Greek words chroma (color) and graphein (to write) because the separated pigments produced distinct bands of color in the column.

  - Tsvet’s technique involved a solid stationary phase (calcium carbonate column) and a liquid mobile phase (solvent mixture) that carried different pigments at different speeds.

- Principle of Chromatography:

  - A mixture is separated into individual components based on differences in their affinity for a stationary phase (solid or liquid-coated surface) and a mobile phase (liquid or gas).

  - Components with higher affinity for the stationary phase move more slowly, while those that favor the mobile phase move faster

Types of Chromatography

### Column Chromatography (Original Technique)

- Uses a packed column containing a solid stationary phase such as silica or alumina.

- A liquid mobile phase flows through the column, separating components based on their relative affinity for the stationary versus mobile phase.

- Compounds that interact more strongly with the stationary phase elute more slowly.

- Serves as the foundational technique from which modern methods such as HPLC and affinity chromatography were developed.

### Thin Layer Chromatography (TLC)

- Uses a thin silica- or alumina-coated plate as the stationary phase to separate small molecules.

- Separation occurs by capillary action as the mobile phase moves up the plate.

- Components are visualized using UV light or chemical stains, and separation is quantified using the retention factor (Rf).

- Commonly used for rapid qualitative analysis, reaction monitoring, and compound identification.

### Gas Chromatography (GC)

- Separates volatile and thermally stable compounds using an inert carrier gas such as helium or nitrogen.

- Components are separated based on differences in volatility and interaction with the column’s stationary phase.

- Provides high resolution and sensitivity but requires analytes to be vaporized.

- Widely used in forensic science, food safety, environmental analysis, and drug testing.

### High-Performance Liquid Chromatography (HPLC)

- Uses high pressure to force liquid samples through a tightly packed column with small stationary-phase particles.

- Provides high-resolution, quantitative separation of biomolecules, pharmaceuticals, and complex mixtures.

- Can be coupled with detectors such as UV–Vis, fluorescence, or mass spectrometry.

- More precise and reproducible than column chromatography but requires specialized instrumentation.

### Affinity Chromatography

- Separates and purifies biomolecules based on highly specific biological interactions such as antibody–antigen or enzyme–substrate binding.

- The target molecule selectively binds to the stationary phase while other components are washed away.

- Enables highly selective purification in a single step.

- Commonly used in biotechnology, protein purification, and biosensor development.

### Chromatography Summary and Comparison

Chromatography is a powerful separation technique that relies on differential interactions between analytes, a stationary phase, and a mobile phase. Simpler techniques such as column chromatography and TLC are inexpensive and easy to perform but offer limited resolution and are primarily qualitative. Gas chromatography provides excellent separation and sensitivity for volatile compounds but is restricted to thermally stable analytes. HPLC offers high resolution, quantitative analysis, and broad applicability but requires specialized equipment and higher operational costs. Affinity chromatography provides unmatched specificity and purity for biomolecules but depends on the availability of selective binding ligands and is not suitable for all analytes. Together, these techniques span a range of speed, selectivity, resolution, and complexity, allowing researchers to choose the most appropriate method based on the sample type and analytical goal.

- Chromatographic Parameters:

  - Retention Time (tR) – The time a compound takes to pass through the column.

  - Retention Factor (Rf​) – The ratio of how far a substance moves relative to the solvent front in TLC.  A higher Rf value indicates weaker interaction with the stationary phase and stronger affinity for the mobile phase.

  - Peak Resolution – The ability to distinguish between two compounds in chromatographic analysis.

- Applications of Chromatography:

  - Pharmaceuticals – Drug purity testing and pharmacokinetics studies.

  - Food Safety – Detecting contaminants like pesticides and toxins.

  - Forensic Science – Identifying drugs and explosives.

  - Environmental Analysis – Detecting pollutants in air and water samples.

### Review Questions

- Who developed chromatography, and what was its original purpose?

- How does column chromatography differ from modern chromatography techniques?

- What is the function of the mobile phase vs. stationary phase in chromatography?

- Why do different compounds have different retention times in a chromatographic system?

- How does gas chromatography (GC) separate volatile compounds?

- What role does high pressure play in HPLC, and why is it useful for biomolecule analysis?

- How can chromatography be used in food safety and environmental analysis?

- What does the retention factor (Rf​) indicate in TLC?

- Why are polarity and solubility important factors in chromatographic separation?

- Describe the process of performing a basic column chromatography experiment and explain how the separated components appear.

- If compound A has Rf = 0.8 and compound B has Rf = 0.3, which interacts more strongly with the stationary phase and why?

# 7. Enzymes in Biosensors: Function, Factors, and Kinetics

## Key Concepts

### Enzyme Function in Biosensors

- Enzymes are biological catalysts that speed up reactions by lowering activation energy, increasing reaction rates without being consumed.

- Key Terms:

  - Substrate – The molecule that the enzyme acts upon.

  - Enzyme-Substrate Complex – A temporary intermediate formed when an enzyme binds to its substrate.

  - Product – The final molecule produced after the enzymatic reaction.

- Enzymatic Biosensors:

  - Used to detect analytes like glucose, lactate, cholesterol, and urea based on enzymatic reactions.

  - Enzymes are immobilized onto a transducer surface (e.g., electrode) to generate a measurable signal.

  - The enzyme’s activity correlates with the concentration of the target analyte.

## Factors Affecting Enzyme Function

- Temperature:

  - Each enzyme functions best within a specific optimal temperature range.

  - High temperatures can cause denaturation, permanently altering the enzyme structure.

  - Low temperatures slow down reaction rates due to decreased molecular motion.

- pH:

  - Each enzyme has an optimal pH range for maximum activity.

  - Extreme pH values can alter the active site structure, reducing enzyme efficiency.

- Heavy Metals & Inhibitors:

  - Heavy metals (e.g., lead (Pb²⁺), mercury (Hg²⁺)) disrupt enzyme function by binding to essential amino acids at the active site.

  - Competitive Inhibition – An inhibitor competes with the substrate for the active site.

  - Non-Competitive (Allosteric) Inhibition – The inhibitor binds to a different site, altering enzyme conformation and reducing activity.

- Cofactors & Coenzymes:

  - Some enzymes require additional molecules to function properly.

  - Cofactors (inorganic molecules):

    - Metalloenzymes require metal ions for activation (e.g., superoxide dismutase (Cu²⁺, Zn²⁺), cytochrome oxidase (Fe²⁺, Cu²⁺)).

  - Coenzymes (organic molecules, often vitamins):

    - NAD⁺/NADH – Required for oxidoreductase enzymes like dehydrogenases.

    - Thiamine pyrophosphate (TPP) – Required for decarboxylases.

    - FAD/FMN – Required for flavoproteins involved in electron transfer.  FAD is tightly associated with glucose oxidase through non-covalent bounds and acts as an electron acceptor in the oxidation of glucose to gluoconic acid.

## Enzyme Kinetics (Michaelis-Menten Model & Lineweaver-Burk Plot)

- Michaelis-Menten Equation:
V = (Vmax * [S]) / (Km + [S])

where:

  - V = Reaction velocity

  - Vmax = Maximum reaction rate when all enzyme active sites are occupied

  - Km = Michaelis constant (substrate concentration at half-maximal velocity)

  - [S] = Substrate concentration

- Interpretation of Km and Vmax:

  - Lower Km → Higher enzyme affinity (stronger substrate binding).

  - Higher Km → Lower enzyme affinity (weaker substrate binding).

  - Higher Vmax → Faster overall reaction rate.

- Turnover Number (kcat):

  - kcat = Vmax / [E]

  - Represents how many substrate molecules an enzyme converts per second.

  - Higher kcat = More efficient enzyme.

- Catalytic Efficiency:

  - kcat / Km determines enzyme efficiency in biosensors.

  - Used to compare different enzymes or mutant variants.

- Reading a Michealis-Menten Plot:

  - Vmax is the plateau value

  - Km is the substrate concentration at V = ½Vmax

### Using the Lineweaver-Burk Plot to Extract Kinetic Parameters

- Lineweaver-Burk Transformation:
1/V = (Km / Vmax) * (1/[S]) + (1/Vmax)

  - Converts the Michaelis-Menten equation into a linear form.

  - X-axis: 1/[S] (Inverse of substrate concentration).

  - Y-axis: 1/V (Inverse of reaction velocity).

- Reading a Lineweaver-Burk Plot:

  - Intercept on Y-axis (1/Vmax) → Inverse of maximum reaction rate.

  - Intercept on X-axis (-1/Km) → Inverse of Michaelis constant (substrate affinity).

  - Slope (Km / Vmax) → Determines reaction efficiency.

- Advantages of Lineweaver-Burk Analysis:

  - Allows estimation of Vmax and Km from experimental data.

  - Helps differentiate competitive vs. non-competitive inhibitors based on changes in slope and intercept.

  - Provides insight into enzyme efficiency for biosensor applications.

## Applications of Enzymes in Biosensors

### Historical Development of Glucose Biosensors

- Clark and Lyons' First Glucose Sensor (1962)

  - The concept of enzyme-based glucose biosensors originated from Leland C. Clark Jr. (the "Father of Biosensors") and Champ Lyons.

  - They developed an electrochemical glucose sensor by immobilizing glucose oxidase (GOx) on an oxygen electrode.

  - The sensor functioned by monitoring the oxygen consumption during the enzymatic oxidation of glucose.

  - This approach was based on the following reaction:

Glucose + O₂ → (Glucose Oxidase) → Gluconic Acid + H₂O₂

  - The decrease in oxygen concentration or the production of hydrogen peroxide (H₂O₂) was correlated to glucose levels.

### Brief Timeline of Commercial Glucose Sensors

- 1973: First Commercial Glucose Sensor

  - Yellow Springs Instrument (YSI) introduced an enzyme-based glucose sensor, primarily for laboratory use.

- 1980s: First Fingerstick Blood Glucose Monitors

  - Medisense and other companies developed disposable glucose test strips, utilizing ferrocene-based mediators instead of oxygen.

- 1999: First Continuous Glucose Monitoring (CGM) System

  - MiniMed (later acquired by Medtronic) released the first subcutaneous glucose sensor for diabetes management.

- 2017-Present: Modern Non-Invasive and Implantable CGMs

  - Devices like FreeStyle Libre (Abbott) and Dexcom G6 use wired-enzyme technology or direct electron transfer mechanisms to provide continuous glucose monitoring without fingersticks.

### Modern Enzyme-Based Biosensors and Applications

- Glucose Biosensors:

  - Use glucose oxidase (GOx) to detect glucose concentration by measuring hydrogen peroxide production or direct electron transfer.

  - Found in diabetes management devices, wearable monitors, and implantable CGMs.

- Lactate Biosensors:

  - Use lactate oxidase to monitor metabolic activity and sports performance.

  - Applied in critical care settings to detect lactic acidosis.

- Cholesterol Biosensors:

  - Use cholesterol oxidase to assess lipid levels in blood.

  - Used for cardiovascular disease monitoring and research applications.

- Urea Sensors:

  - Use urease to measure ammonia formation, aiding in kidney function assessment.

  - Found in renal disease diagnostics and dialysis monitoring.

- Electrochemical Enzymatic Biosensors:

  - Use direct electron transfer or mediators (ferrocene, quinones, osmium complexes) to enhance signal detection.

  - Enzymes are often immobilized on conductive surfaces like gold electrodes or carbon nanotubes for improved sensor stability and response time.

## Enzyme Classification System

- Enzymes are classified using the Enzyme Commission (EC) number system, where the first digit refers to the broad class of enzyme reaction:

  - EC 1 – Oxidoreductases (e.g., glucose oxidase) – Catalyze oxidation-reduction reactions.

  - EC 2 – Transferases – Transfer functional groups between molecules.

  - EC 3 – Hydrolases – Break bonds via hydrolysis.

  - EC 4 – Lyases – Cleave bonds without hydrolysis or oxidation.

  - EC 5 – Isomerases – Catalyze structural rearrangements within a molecule.

  - EC 6 – Ligases – Join two molecules using ATP.

## Review Questions

- What is an enzyme, and how does it function in a biosensor?

- How do cofactors and coenzymes assist enzyme function?

- What is the difference between a competitive and non-competitive inhibitor?

- How does temperature impact enzyme function, and does excessive heat cause denaturation?

- What does the Km value indicate about enzyme affinity?

- How can enzyme kinetics be determined using a Lineweaver-Burk plot?

- How does substrate concentration influence enzymatic biosensor output?

- Explain the step-by-step method of using a Lineweaver-Burk plot to determine enzyme kinetic parameters.

- Who were Clark and Lyons, and what was their contribution to biosensor technology?

- How did the first glucose sensor (1962) work, and what was its basic principle?

- What are the major differences between first-generation, second-generation, and third-generation glucose sensors?

- What are the advantages of using direct electron transfer (DET) or wired-enzyme technology in glucose biosensors?

- Why was the introduction of continuous glucose monitoring (CGM) sensors a major advancement in diabetes management?

- What is the chemical reaction catalyzed by glucose oxidase (GOx) in a glucose biosensor?

- How does the concentration of oxygen affect the performance of first-generation glucose biosensors?

- What does the first number in an Enzyme Commission (EC) classification represent?

- Which enzyme class would be used in biosensors that measure redox-active compounds, and why?

- Given a Michaelis–Menten or Lineweaver–Burk plot, identify which enzyme has the highest affinity and which has the highest Vmax.

### 8. Mediators in Electrochemical Biosensors

#### Key Concepts

- Mediators are small molecules that facilitate electron transfer between an enzyme and an electrode, enabling efficient signal generation in biosensors.

- Why Mediators Are Needed:

  - Some oxidoreductase enzymes, such as glucose oxidase (GOx) and lactate oxidase, generate electrons but cannot efficiently transfer them directly to an electrode.

  - Mediators serve as electron shuttles, bypassing the need for direct enzyme-electrode interaction and improving biosensor sensitivity.

  - Without mediators, biosensors relying on oxygen-dependent redox reactions may suffer from limited performance due to fluctuations in oxygen availability.

  - Mediator-based systems are preferred in continuous glucose monitors (CGMs) because they reduce oxygen dependence and allow operation at lower potentials, minimizing interference.

- Common Mediators in Enzymatic Biosensor (You do not need to know the different types of mediators, but here are some examples for your reference).

  - Ferrocene and Ferrocene Derivatives – Stable redox compounds that allow rapid electron transfer at low potential, widely used in glucose biosensors.

  - Quinones – Efficient electron carriers that participate in biological redox reactions.

  - Osmium Complexes – Highly stable mediators used in wired-enzyme biosensors, offering enhanced biocompatibility and efficiency.

#### How Mediators Function in a Biosensor

- Enzyme Catalysis – The oxidoreductase enzyme catalyzes the oxidation of the analyte (e.g., glucose).

- Electron Transfer to Mediator – The enzyme transfers electrons to the mediator instead of directly to the electrode.

- Mediator Transfers Electrons to Electrode – The mediator undergoes oxidation or reduction at the electrode surface, generating a measurable current.

#### Advantages of Using Mediators in Electrochemical Biosensors

- Facilitates Electron Transfer – Allows enzymes to transfer electrons even if they are not in direct contact with the electrode.

- Reduces Voltage Requirements – Lowers the working potential required for enzyme activation, minimizing interference from other electroactive species.

- Increases Sensor Stability – Mediators provide consistent performance, reducing dependency on oxygen levels for oxidation reactions.

- Improves Sensitivity and Response Time – Mediator-based biosensors generally have faster reaction kinetics than oxygen-dependent biosensors.

#### Understanding the Difference Between Electron Transfer Pathways:

- Oxygen-Dependent (Unmediated) Electron Transfer:

  - In traditional oxidase enzyme reactions, the enzyme transfers electrons directly to molecular oxygen (O₂), producing hydrogen peroxide (H₂O₂) as a byproduct.

  - For example, in the case of glucose oxidase (GOx), the reaction follows:

    - Glucose + O₂ → (GOx) → Gluconic Acid + H₂O₂

  - Oxygen is the terminal electron acceptor, meaning that glucose oxidase remains dependent on oxygen levels in the environment.

- Mediator-Based Electron Transfer:

  - In a mediator system, the enzyme transfers electrons to a redox mediator (M) instead of O₂.

  - The reaction follows:

    - Glucose + Mediator (Ox) → (GOx) → Gluconic Acid + Mediator (Red)

  - The reduced mediator then transfers electrons to the electrode, bypassing the need for oxygen.

  - Some oxygen may still be consumed if the mediator concentration is low or if O₂ competes with the mediator for electron uptake.

- Direct Electron Transfer (DET):

  - In DET, the enzyme is wired directly to the electrode, meaning it does not need O₂ or a mediator to complete the redox cycle.

  - The reaction occurs as follows:

    - Glucose + GOx (Ox) → Gluconic Acid + GOx (Red)

    - GOx (Red) → GOx (Ox) + e⁻

  - The enzyme transfers electrons directly to the electrode, eliminating the need for oxygen completely.

  - Since oxygen is bypassed, no H₂O₂ is generated, and oxygen is not consumed in the reaction.

### Comparison of Electron Transfer Mechanisms in Electrochemical Biosensors

| Feature | Un-Mediated Electron Transfer (e.g., Oxidation of H₂O₂ at Electrode Surface) | Mediator-Based Electron Transfer | Direct Electron Transfer (DET) (Oxygen Not Required with Oxidase Enzymes) |
| --- | --- | --- | --- |
| Electron Transfer Efficiency | Low to Moderate (depends on electrode material and reaction kinetics) | High (mediators facilitate rapid electron shuttling) | Moderate (requires enzyme-electrode proximity) |
| Working Voltage | High (oxidation of H₂O₂ typically requires >0.6V) | Lower (mediators reduce voltage requirement) | Moderate (depends on enzyme-electrode interaction) |
| Dependency on Oxygen | High (requires oxidation of H₂O₂, which is generated by oxidase enzymes using oxygen) | Low (mediators shuttle electrons independently of oxygen) | None (direct electron transfer between enzyme and electrode) |
| Reaction Speed | Slower (reaction limited by H₂O₂ diffusion and electrode activity) | Faster (mediators enable rapid redox cycling) | Moderate (enzyme-electrode interaction limits rate) |
| Sensor Stability | Moderate (electrode fouling can occur due to side reactions) | High (mediators improve long-term stability) | Variable (enzyme orientation and stability affect performance) |
| Selectivity | Lower (other electroactive species may interfere) | Higher (mediators can be optimized for specificity) | Moderate (depends on enzyme-electrode interaction) |
| Common Applications | Peroxide-based biosensors, classical amperometric glucose sensors | Modern glucose and lactate biosensors (ferrocene, osmium mediators) | Direct electron transfer glucose sensors, wired enzyme biosensors |

### Key Takeaways from Comparison:

- Un-Mediated Electron Transfer relies on the direct oxidation of hydrogen peroxide at the electrode, but it requires a high working voltage, making it susceptible to interference from other electroactive species.

- Mediator-Based Electron Transfer significantly reduces the working voltage and improves electron transfer efficiency by acting as an electron shuttle between the enzyme and electrode.

- Direct Electron Transfer (DET) eliminates the need for oxygen-dependent reactions but requires optimized enzyme immobilization to ensure proper electronic communication with the electrode.

## Review Questions

- What is the primary role of mediators in electrochemical biosensors, and how do they facilitate electron transfer?

- Why do some oxidoreductase enzymes require mediators for efficient electron transfer instead of directly transferring electrons to the electrode?

- How does a mediator improve biosensor stability and reliability compared to direct electron transfer (DET)?

- Why do mediator-based biosensors require a lower operating voltage than oxygen-dependent biosensors, and how does this benefit sensor performance?

- Why do continuous glucose monitoring (CGM) sensors often use mediator-based electron transfer instead of oxygen-dependent reactions?

- How does mediator concentration affect oxygen consumption in a biosensor system?

- What are the differences between un-mediated electron transfer, mediator-based transfer, and direct electron transfer (DET)?

- Why is electron transfer efficiency higher in mediator-based biosensors compared to hydrogen peroxide-based detection?

# 9. Electrochemical Techniques: Cyclic Voltammetry, Electrochemical Impedance Spectroscopy, and the Electrochemical Double Layer

## Key Concepts

### Electrochemical Reactions & Standard Potential

Voltammetry

- Reversible Reactions exhibit symmetric oxidation and reduction peaks in cyclic voltammetry (CV), indicating fast electron transfer kinetics.

- Irreversible Reactions display peak separation and distorted CV shapes, signifying slow charge transfer rates.

- Standard Potential (E⁰) represents the equilibrium potential of a redox couple and can be estimated using the midpoint of oxidation and reduction peaks in a cyclic voltammogram.

- Faradaic vs. Non-Faradaic Processes:

  - Faradaic processes involve redox reactions, where electrons transfer between the electrode and analyte.

  - Non-Faradaic processes involve capacitive charging, such as charge buildup in the electrochemical double layer, without net electron transfer.

- Electron Transfer Kinetics depend on diffusion, activation energy, and electrode surface modifications.

- Mass Transport Effects influence peak currents in CV, consisting of diffusion, convection, and migration.

- Diffusion-Controlled Current follows the Randles-Sevcik equation, where peak current (Ip) is proportional to the square root of scan rate in diffusion-limited systems.

Noting reversibility on CV plots

A reversible redox reaction typically has:
– Symmetric anodic and cathodic peaks
– Small peak separation (~59 mV/n at 25°C for a one-electron transfer)
– Peak current proportional to √(scan rate)

### Randles Circuit & Nyquist Plot in Electrochemical Impedance Spectroscopy (EIS)

- Electrochemical Impedance Spectroscopy (EIS) is a technique that measures the resistance and capacitance of an electrochemical system across different frequencies, helping analyze charge transfer and diffusion effects.

- Randles Circuit is an equivalent electrical model for an electrochemical system, composed of:

  - Solution Resistance (Rs): Resistance of the electrolyte solution between the working and reference electrodes.

  - Charge Transfer Resistance (Rct): Resistance at the electrode-solution interface; a key indicator of biosensor efficiency.

  - Double-Layer Capacitance (Cdl): Represents charge storage at the electrode-electrolyte interface.

  - Warburg Impedance (Zw): Models diffusion effects at low frequencies, where ion transport in solution dominates.

- Nyquist Plot Interpretation:

  - High Frequencies: Dominated by solution resistance (Rs), appearing as the x-axis intercept.

  - Mid-Frequencies: The semi-circle diameter represents charge transfer resistance (Rct), associated with electron transfer at the electrode surface.

  - Low Frequencies: The linear Warburg region appears, reflecting diffusion-controlled impedance.

- Nyquist plot “how to read”

  - In a Nyquist plot:
– Left x-intercept = Rs
– Semicircle diameter = Rct
– 45° line at low frequency = Warburg impedance (diffusion)

### The Electrochemical Double Layer and Electrode Polarization

- Electrochemical Double Layer (EDL) forms at the electrode/electrolyte interface, consisting of:

  - Inner Helmholtz Layer (Stern Layer): Adsorbed ions tightly bound to the electrode surface.

  - Outer Helmholtz Layer: A secondary layer of solvated ions interacting with the inner layer.

  - Diffuse Layer: A cloud of ions extending into the bulk solution, forming a charge gradient.

- Importance in Electrochemical Sensing:

  - The double layer affects charge transfer resistance and impacts electron kinetics in biosensors.

  - Electrode surface modifications (e.g., nanoparticles, polymers, biomolecules) alter double-layer capacitance (Cdl), affecting sensor performance.

  - Non-Faradaic sensing techniques, such as capacitive biosensors, rely on changes in double-layer properties instead of redox reactions.

- Electrode Polarization:

  - Applying an external voltage to an electrode polarizes it, shifting ion distributions in the double layer.

  - In CV, polarization drives redox reactions, while in EIS, it alters charge transfer resistance (Rct) and Warburg impedance (Zw).

## Review Questions

- How do you determine if an electrochemical reaction is reversible using cyclic voltammetry?

- What factors influence electron transfer rates in electrochemical biosensors?

- How do Faradaic and non-Faradaic processes differ in electrochemical sensing?

- What is the significance of the standard potential (E⁰) in cyclic voltammetry?

- How does mass transport (diffusion, convection, migration) impact biosensor performance?

- How does double-layer capacitance (Cdl) affect charge transfer in electrochemical systems?

- How can you estimate E⁰ and reversibility of a redox reaction using a cyclic voltammogram?

- Why does the Nyquist plot show a semi-circle at mid-frequencies, and what does it represent?

- What is the difference between solution resistance (Rs) and charge transfer resistance (Rct)?  Which is often measured to calibrate electrochemical biosensors and why?

- What does a linear region at low frequencies in a Nyquist plot indicate about mass transport?

- How does EIS differ from CV in studying electrochemical processes?

- Why is Warburg impedance (Zw) important in biosensor applications?

- How does electrode surface modification (e.g., nanoparticles, polymer coatings) affect Randles Circuit parameters?

- What does it mean to polarize an electrode, and why is this important in electrochemical techniques?

- How does the Helmholtz layer differ from the diffuse layer in the electrochemical double layer model?

- How does the double layer contribute to capacitive biosensor signal generation?

- What is the relationship between scan rate and peak current in a diffusion-controlled CV process?

Top of Form

Bottom of Form